Frigate-to-Meshtastic Face Recognition Bridge

AI-First Setup Document for CLI Agent Execution

System Architecture

Frigate (Docker) → MQTT Broker (Mosquitto) → Python Bridge → USB Serial → Meshtastic Node → Mesh Network

Objective

Deploy an automated notification system that sends "Daddy's Home" to a Meshtastic mesh network when Frigate's face recognition system identifies Mark Snow via IP camera.

Prerequisites

- Frigate Docker container running with face recognition enabled

- IP camera configured and recognizing target face

- Meshtastic node connected via USB serial to host laptop

- Docker and Docker Compose installed

- Python 3.8+ with pip available

Implementation Steps

Step 1: Deploy Mosquitto MQTT Broker

Create directory structure for persistent storage[1]:

mkdir -p ~/frigate-mesh/mosquitto/{config,data,log}
cd ~/frigate-mesh

Create ~/frigate-mesh/mosquitto/config/mosquitto.conf:

listener 1883
allow_anonymous true
persistence true
persistence_location /mosquitto/data/
log_dest file /mosquitto/log/mosquitto.log
log_type all

Create docker-compose.yml in ~/frigate-mesh/:

version: "3.8"

services:
mosquitto:
image: eclipse-mosquitto:2
container_name: mosquitto
restart: unless-stopped
ports:
- "1883:1883"
volumes:
- ./mosquitto/config/mosquitto.conf:/mosquitto/config/mosquitto.conf
- ./mosquitto/data:/mosquitto/data
- ./mosquitto/log:/mosquitto/log
networks:
- frigate-network

networks:
frigate-network:
external: true

If frigate-network doesn't exist, create it[2]:

docker network create frigate-network

Launch Mosquitto:

docker-compose up -d

Verify broker is running:

docker logs mosquitto

Should show: "mosquitto version 2.x running"

Step 2: Configure Frigate MQTT Integration

Locate your existing Frigate config.yml. Add or update the MQTT section[3]:

mqtt:
enabled: true
host: mosquitto # Use container name if on same Docker network

If Frigate is NOT on frigate-network, use host IP: 192.168.x.x

port: 1883
topic_prefix: frigate
client_id: frigate

If Frigate is on a different Docker network, connect it to frigate-network:

docker network connect frigate-network frigate

Restart Frigate:

docker restart frigate

Verify MQTT events are publishing:

docker exec mosquitto mosquitto_sub -t "frigate/#" -v

You should see messages on topics like frigate/tracked_object_update when face detection occurs[3].

Step 3: Identify Meshtastic Serial Port

Plug in the Meshtastic node via USB and detect the serial port[4]:

Linux:
ls /dev/ttyUSB* /dev/ttyACM*

Common: /dev/ttyUSB0 or /dev/ttyACM0

macOS:
ls /dev/cu.usbmodem* /dev/cu.usbserial*

Common: /dev/cu.usbmodem14201

Windows:

Check Device Manager → Ports (COM & LPT)

Common: COM3, COM4

Test connection:

meshtastic --port /dev/ttyUSB0 --info

Note the device path and verify you can see node information.

Add user to dialout group (Linux only, then logout/login)[4]:

sudo usermod -aG dialout $USER

Step 4: Install Python Dependencies

Create a virtual environment in ~/frigate-mesh/:

cd ~/frigate-mesh
python3 -m venv venv
source venv/bin/activate # Linux/macOS

venv\Scripts\activate # Windows

Install required packages:

pip install paho-mqtt meshtastic

Step 5: Deploy Python Bridge Script

Create ~/frigate-mesh/frigate_mesh_bridge.py:

#!/usr/bin/env python3
"""
Frigate-to-Meshtastic Bridge
Listens for face recognition events on MQTT and sends mesh notifications
"""

import json
import time
import logging
import paho.mqtt.client as mqtt
import meshtastic.serial_interface

========== CONFIGURATION ==========

MQTT_BROKER = "localhost"
MQTT_PORT = 1883
FRIGATE_TOPIC = "frigate/tracked_object_update"

Meshtastic Configuration

SERIAL_PORT = None # None = auto-detect, or set: "/dev/ttyUSB0", "COM3"
MESSAGE = "Daddy's Home 🏠"
CHANNEL_INDEX = 0 # 0 = primary channel, change for specific group channel

Face Recognition Settings

TARGET_FACE = "Mark" # Must exactly match Frigate Face Library name (case-sensitive)
MIN_CONFIDENCE = 0.80 # Minimum confidence score (0.0-1.0)
COOLDOWN_SECONDS = 300 # 5 minutes between repeat notifications

========== GLOBALS ==========

logging.basicConfig(
level=logging.INFO,
format='%(asctime)s - %(levelname)s - %(message)s'
)
logger = logging.getLogger(name)

last_trigger_time = 0
mesh_interface = None

========== MESHTASTIC CONNECTION ==========

def connect_meshtastic():
"""Establish serial connection to Meshtastic node"""
global mesh_interface

logger.info("Connecting to Meshtastic node via serial...")
try:
    if SERIAL_PORT:
        mesh_interface = meshtastic.serial_interface.SerialInterface(
            devPath=SERIAL_PORT
        )
    else:
        mesh_interface = meshtastic.serial_interface.SerialInterface()
    
    node_info = mesh_interface.getMyNodeInfo()
    logger.info(f"Connected to Meshtastic node: {node_info}")
    return True
except Exception as e:
    logger.error(f"Failed to connect to Meshtastic: {e}")
    return False

========== MQTT CALLBACKS ==========

def on_connect(client, userdata, flags, rc, properties=None):
"""Callback when MQTT broker connection established"""
if rc == 0:
logger.info("Connected to MQTT broker")
client.subscribe(FRIGATE_TOPIC)
logger.info(f"Subscribed to {FRIGATE_TOPIC}")
else:
logger.error(f"MQTT connection failed with code {rc}")

def on_message(client, userdata, msg):
"""Callback when MQTT message received"""
global last_trigger_time

try:
    payload = json.loads(msg.payload.decode())
except json.JSONDecodeError as e:
    logger.warning(f"Invalid JSON payload: {e}")
    return

        # Filter for face recognition events only
if payload.get("type") != "face":
    return

        face_name = payload.get("name")
confidence = payload.get("score", 0)
camera = payload.get("camera", "unknown")

        logger.debug(f"Face detected: {face_name} (score: {confidence:.2f}, camera: {camera})")

        # Check if target face with sufficient confidence
if face_name == TARGET_FACE and confidence >= MIN_CONFIDENCE:
    current_time = time.time()
    
    # Enforce cooldown period
    if current_time - last_trigger_time < COOLDOWN_SECONDS:
        remaining = int(COOLDOWN_SECONDS - (current_time - last_trigger_time))
        logger.info(
            f"Face matched but cooldown active ({remaining}s remaining)"
        )
        return
    
    # Update trigger time and send mesh message
    last_trigger_time = current_time
    logger.info(
        f"✅ {face_name} recognized with {confidence:.2f} confidence"
    )
    
    try:
        mesh_interface.sendText(MESSAGE, channelIndex=CHANNEL_INDEX)
        logger.info(f"Mesh message sent: '{MESSAGE}' on channel {CHANNEL_INDEX}")
    except Exception as e:
        logger.error(f"Failed to send mesh message: {e}")

========== MAIN ==========

def main():
"""Main execution loop"""
global mesh_interface

# Connect to Meshtastic node
if not connect_meshtastic():
    logger.error("Exiting: Cannot establish Meshtastic connection")
    return

        # Setup MQTT client
mqtt_client = mqtt.Client(mqtt.CallbackAPIVersion.VERSION2)
mqtt_client.on_connect = on_connect
mqtt_client.on_message = on_message

        try:
    mqtt_client.connect(MQTT_BROKER, MQTT_PORT, 60)
    logger.info("Bridge service running. Press Ctrl+C to stop.")
    mqtt_client.loop_forever()
except KeyboardInterrupt:
    logger.info("Shutting down bridge service...")
except Exception as e:
    logger.error(f"MQTT connection error: {e}")
finally:
    if mesh_interface:
        mesh_interface.close()
        logger.info("Meshtastic connection closed")

if name == "main":
main()

Make the script executable:

chmod +x frigate_mesh_bridge.py

Step 6: Configure and Test

Update configuration variables in frigate_mesh_bridge.py:

- TARGET_FACE: Set to exact name from Frigate Face Library (case-sensitive)

- SERIAL_PORT: Set to detected port from Step 3, or leave None for auto-detect

- CHANNEL_INDEX: Set to specific Meshtastic channel index if not using primary (0)

- MESSAGE: Customize notification text

- MIN_CONFIDENCE: Adjust threshold (0.80 = 80% confidence)

- COOLDOWN_SECONDS: Adjust notification frequency limit

Run the bridge:

cd ~/frigate-mesh
source venv/bin/activate
python frigate_mesh_bridge.py

Expected output:

2026-03-25 07:30:15 - INFO - Connecting to Meshtastic node via serial...
2026-03-25 07:30:17 - INFO - Connected to Meshtastic node: {...}
2026-03-25 07:30:17 - INFO - Connected to MQTT broker
2026-03-25 07:30:17 - INFO - Subscribed to frigate/tracked_object_update
2026-03-25 07:30:17 - INFO - Bridge service running. Press Ctrl+C to stop.

Test face detection:

- Walk in front of IP camera

- Monitor bridge logs for face recognition event

- Check Meshtastic devices for "Daddy's Home" message

Step 7: Create Systemd Service (Optional - Linux)

For automatic startup on boot, create /etc/systemd/system/frigate-mesh-bridge.service:

[Unit]
Description=Frigate Meshtastic Bridge
After=network.target docker.service
Requires=docker.service

[Service]
Type=simple
User=YOUR_USERNAME
WorkingDirectory=/home/YOUR_USERNAME/frigate-mesh
ExecStart=/home/YOUR_USERNAME/frigate-mesh/venv/bin/python /home/YOUR_USERNAME/frigate-mesh/frigate_mesh_bridge.py
Restart=on-failure
RestartSec=10

[Install]
WantedBy=multi-user.target

Replace YOUR_USERNAME with your actual username. Enable and start:

sudo systemctl daemon-reload
sudo systemctl enable frigate-mesh-bridge
sudo systemctl start frigate-mesh-bridge
sudo systemctl status frigate-mesh-bridge

Verification Checklist

- Mosquitto container running and accessible on port 1883

- Frigate publishing MQTT events to frigate/tracked_object_update

- Meshtastic node detected and connected via USB serial

- Python dependencies installed (paho-mqtt, meshtastic)

- Bridge script runs without errors

- Face recognition triggers mesh notification

- Cooldown period prevents message spam

- (Optional) Systemd service enables automatic startup

Troubleshooting

| Issue | Solution |
| --- | --- |
| Mosquitto container fails | Check port 1883 not in use: sudo netstat -tulpn ¦ grep 1883 |
| Frigate not publishing MQTT | Verify mqtt.host resolves from Frigate container; use host IP |
| Serial port permission denied | Add user to dialout group: sudo usermod -aG dialout \$USER |
| Meshtastic auto-detect fails | Explicitly set SERIAL_PORT variable in bridge script |
| Face events but no mesh msg | Verify TARGET_FACE matches Frigate name exactly (case-sensitive) |
| MQTT connection refused | Ensure Frigate and Mosquitto on same Docker network |
| Message sent to wrong channel | Adjust CHANNEL_INDEX to target specific Meshtastic channel |

Table 1: Common issues and resolution steps

Advanced Configuration

Multi-Face Recognition

To trigger different messages for different people, modify the on_message callback:

FACE_MESSAGES = {
"Mark": "Daddy's Home 🏠",
"Alice": "Mom's Here 👋",
"Charlie": "Charlie Arrived 🎉"
}

In on_message callback:

if face_name in FACE_MESSAGES and confidence >= MIN_CONFIDENCE:
message = FACE_MESSAGES[face_name]
mesh_interface.sendText(message, channelIndex=CHANNEL_INDEX)

Private Channel Messaging

To send to a specific Meshtastic channel (e.g., family channel at index 1):

CHANNEL_INDEX = 1 # Set to your target channel index

List available channels on your node:

meshtastic --ch-index 0
meshtastic --ch-index 1

etc.

MQTT Authentication

For production deployments with authentication, update mosquitto.conf:

listener 1883
allow_anonymous false
password_file /mosquitto/config/passwd

Create password file:

docker exec mosquitto mosquitto_passwd -c /mosquitto/config/passwd frigate

Enter password when prompted

Update Frigate config.yml:

mqtt:
host: mosquitto
port: 1883
user: frigate
password: your_secure_password

Update bridge script:

mqtt_client.username_pw_set("frigate", "your_secure_password")

System Requirements

- CPU: x86_64 or ARM64 (Raspberry Pi 4+ supported)

- RAM: 512MB minimum for bridge script (Frigate requires 2GB+)

- Storage: 100MB for dependencies + Mosquitto data

- Network: LAN connectivity for Frigate IP camera access

- USB: One available USB port for Meshtastic serial connection

Security Considerations

- Face recognition data: Frigate stores facial embeddings locally in Docker volumes

- MQTT traffic: Currently unencrypted; enable TLS for production deployments

- Meshtastic mesh: Messages broadcast to all mesh participants on selected channel

- Serial access: Ensure only authorized users have dialout group membership

- FERPA compliance: This system processes student images if deployed in school context; review privacy policies[5]

Performance Optimization

- Confidence threshold: Increase MIN_CONFIDENCE to 0.90+ to reduce false positives

- Cooldown period: Adjust COOLDOWN_SECONDS based on arrival frequency patterns

- MQTT QoS: Default QoS 0 (fire-and-forget); increase to QoS 1 for reliability

- Meshtastic LoRa settings: Use SF7 for faster transmission if range permits

Integration with Existing Operator Framework

To integrate with your Operator emergency response system, publish additional MQTT topics:

In on_message callback after successful face match:

emergency_payload = {
"event": "arrival",
"person": face_name,
"timestamp": time.time(),
"camera": camera,
"confidence": confidence
}

mqtt_client.publish(
"operator/events/arrival",
json.dumps(emergency_payload)
)

This enables cross-platform event correlation between face recognition and mesh-based emergency systems.

Future Enhancements

- Integrate with Liberty Mesh token economics for message priority routing

- Add LLM-based contextual message generation using Lemonade infrastructure

- Implement PUF-based cryptographic signatures for message authentication

- Deploy compressed embeddings over LoRa using chirp spread spectrum optimization

- Build Trenton Makes dashboard for face recognition analytics

References

[1] Cedalo. (2025). Mosquitto Docker Configuration - Ultimate Guide. https://cedalo.com/blog/mosquitto-docker-configuration-ultimate-guide/

[2] Blake Blackshear. (2022). Example setup of an MQTT server in docker-compose. GitHub Discussions. https://github.com/blakeblackshear/frigate/discussions/3265

[3] Frigate Documentation. (n.d.). MQTT Integration. https://docs.frigate.video/integrations/mqtt

[4] Meshtastic Documentation. (n.d.). Using the Meshtastic Python Library. https://meshtastic.org/docs/development/python/library/

[5] U.S. Department of Education. Family Educational Rights and Privacy Act (FERPA). https://www2.ed.gov/policy/gen/guid/fpco/ferpa/